07/11/2019

Development Services

220 SE Green Street

Lee’s Summit, MO						       APPLICATION # PL2019198

64063

Re:	RESPONSE TO COMMENTS PROVIDED BY PLANNING, ENGINEERING AND TRAFFIC FOR AISLES ONLINE KIOSK AT HY-VEE LOCATED AT 310 SW WARD ROAD, LEE’S SUMMIT, MO 64081

Fire Review Jim Eden Assistant Chief Approved with Conditions (816) 969-1303 Jim.Eden@cityofls.net

- All issues pertaining to life safety and property protection from the hazards of fire, explosion or dangerous conditions in new and existing buildings, structures and premises, and to the safety to fire fighters and emergency responders during emergency operations, shall be in accordance with the 2018 International Fire Code.

Response: Noted.

Planning Review Jennifer Thompson Planner Corrections (816) 969-1239 Jennifer.Thompson@cityofls.net

- The elevations need to be revised to address some architectural concerns of the proposed building.

- This building should be more compatible and complimentary to the adjacent gas station.  Revise the columns of the canopy to be brick similar to the gas station.

Response: Brick banding details have been added to building pilasters. The exterior canopy columns have been wrapped in brick to mimic the gas canopy, however, the interior columns remain painted.  The minimal thickness is desired to decrease the security risk of a wide column so close to the entry point. They will be painted to match the neutral color.

- Revise the brick pilasters to give the brick columns some depth and articulation.

Response: the pilasters have a thickness of approximately ½” to create desired articulations.

- Include faux windows on the front elevation.

Response: Faux windows and an additional pilaster was added to the front face of the building.

- Remove the red ACM panels on the main building.  Replace this banding with something similar to the gas station building (stucco banding?).  The red aluminum panels are a conditional material and would require Planning Commission and City Council approval.

Response: The parapet that acts as the RTU screens will be finished with Stuccoflex material instead of metal. The color will coordinate with the c-store colors.

- Expand drive lane area for 3rd stall, it seems a bit tight.

Response: Drive lane area for 3rd stall increased. Turn movements graphically confirmed.

- The proposed wall signs/canopy signs will require separate permit review and approvals.

BMcD Response: Noted.

- How will supplies be delivered and stocked to this site?

Response: Supplies will be delivered 2 times a day by box truck (one early morning delivery, one afternoon). Box truck will be equipped with tommy gate and unloaded at the front door in the south most drive-thru lane by hand.

- It is noted landscaping will be removed in the area in front of the existing fueling canopy area. Any landscaping removed will be required to be replaced.

Response: The area previously called out as “landscaping” is just native grass area. No bushes, plants etc. being removed that would need to be replaced.

- The scale seems off on some of the dimensions on sheet C2.1 (parking stalls and other call outs). Label the dimension of the width of the proposed parking stalls.

Response: Dimensions of proposed parking added. See sheet C2.1. Scales confirmed.

- Provide the manufacturer's specification for all proposed exterior building lighting.

Response: Manufacturer’s specification included.

- Revise the concrete paving detail to meet the standards set forth in the Unified Development Ordinance, Article 8.

Response: Concrete paving detail updated to meet UDO Article 8. See detail 3 on sheet C5.0

- Reference and provide CG-1 curbing within the detail.

Response: CG-1 curb detail added.

- Reference the Traffic Engineering comment regarding sidewalks.  Revise the plans to show this connection.

- Will there be roof-top units or ground-mounted mechanical units?  Please clarify and provide the method of screening.

Response: there will be a roof top unit that will be screened by the parapet. See building elevations.

Engineering Review Gene Williams Senior Staff Engineer Corrections (816) 969-1223 Gene.Williams@cityofls.net

- Will any portion of the canopy extend within the sanitary sewer easement?  No encroachment into the easement is allowed, either by the canopy, or the columns, or the footings for the columns.

Response: Design revised to ensure no conflict with easements.

- Sheet C2.1:  Cross-hatch pattern is shown, and it appears this is pavement marking?

Response: Cross hatch is pavement marking. Hatching now called out on C2.1

- Detail Sheets:  SAN-1 standard detail is missing, and is required.

Response: SAN-1 standard detail has been added. See sheet C5.2

- Sheet C5.0:  If using PCC for pavement, the standard design from the Unified Development Ordinance (UDO) requires a minimum of 4 inch aggregate base.  In addition, the detail showing 6 inches beyond the back of curb needs to be revised to a minimum of 12 inches behind the back of curb.

Response: Updates have been made to paving and curb & gutter detail. See C5.0.

- It is unclear how the farthest south drive-through lane will function.  It appears the geometry will not allow for safe passage of a vehicle.

Response: Geometry has been modified to improve function of south drive-thru lane.

- An erosion and sediment control plan was missing.  Even though this project is relatively small, an erosion and sediment control plan is required for all projects with disturbed areas greater than 2000 sq. feet.

Response: Erosion control included with grading plan. Refer to sheet C.3.0

- Sheet C1.0: If removing the crosswalk as shown, then a new sidewalk is recommended along the north side of the private drive.  Please see comments from the City Traffic Engineer concerning these recommendations.

Response: Sidewalk added per Traffic Engineers recommendations. See C2.1

- An itemized and sealed Engineer's Estimate of Probable Construction Costs for sitework is required as part of the final submittal drawings.  The Engineering Plan Review and Inspection Fee is based on this estimate of sitework, and calculated at 3% of the sitework.  Items to include in the estimate are: 1) demolition, 2) new PCC pavement and sidewalk, 3) aggregate base, including the area one (1) foot beyond the back of curb, 4) curb and gutter, 5) sanitary sewer connection and lateral, 6) water lines and connections, 7) erosion and sediment control measures and devices, and 8) final restoration, including sodding, seeding, fertilizer, mulch, and topsoil.

Response: Estimate provided as requested

Traffic Review Michael Park City Traffic Engineer Corrections

(816) 969-1820 Michael.Park@cityofls.net

- Recommend a sidewalk from the Cedar Creek Xing intersection along the perimeter drive between Hy-Vee Grocery and Hy-Vee C-Store/Service Station and the Hy-Vee C-Store/Service Station driveway to replace the pedestrian route that is removed.  This route provides pedestrian access between land uses/properties without traffic conflict associated with fueling activity.

Response: Sidewalk added per Traffic Engineers recommendations. See C2.1

- Where crosswalk removal is proposed, the associated ramp, sidewalk and path should also be removed, backfilled and sodded.

Response: Noted. Contractor will be required to return ramp to landscape.

Building Codes Review Joe Frogge Plans Examiner (816) 969-1241 Joe.Frogge@cityofls.net

No Comments

Sincerely,

Luo Hamateyo, E.I.T

Civil Engineer

cc:	File