SUMMIT INNOVATION CENTER

PRELIMINARY DEVELOPMENT PLAN UPDATE

Submitted By:

Townsend Summit, LLC

May 11, 2015

(Revised)

INTRODUCTION

Background

This is an update to an approved Preliminary Development Plan.  Summit Technology Campus Preliminary Development Plan was approved by the Lee’s Summit, Missouri City Council on May 20th 1999.  The Summit Technology Campus plan calls for a mix of light-industrial, flex-tech, office and retail uses centered on the renovation of the 1.3 million square feet of buildings, and promotes amenities that drive a mix of employment and commercial activity.  This updated plan is consistent with the approved plan.  The update accounts for many recent developments impacting the property:

- the dedication of Right-of-way and design of the Tudor Road connection to Ward Road;

- the design of Ward Roads future expansion;

- the construction of Innovation Drive between Tudor and Ward Road;

- the development of Summit Faire through an updated plan in the west portion of the plan area,

- the planning and approvals for Summit Place north of this plan; and

- the construction of an education collaboration hub coordinated with the R-7 School District, Central Missouri University known as the Missouri Innovation Campus.

This Missouri Innovation Campus is the anchor of an education, innovation and institution hub reflected in this plan update, which contributes to the overall original vision of the Summit Technology Campus approved in 1999.

The original plan was approved as “a flexible working model to guide and control successful development generally, so that changes in location or use can occur within the framework of the Plan’s standards for maintaining architectural control in terms of aesthetics as regards to light, air, open space, green grass, trees, landscaping, lighting and relationships amongst buildings.”  [Approved Summit Technology Campus plan, Part 2, page 2.].  This plan update executes that original intent and is consistent with all aspects of the approved plan.

Overview of Plan Update

Summit Innovation Center is a collaborative education, innovation and institution hub where education, industry and supporting businesses come together to advance ideas.  The strategic location of this site, the synergy of the government, education, and business partners, and the inspiring design of the buildings and spaces in the Summit Innovation Center drive its mission to further the business and education interests of its stakeholders.

This revised preliminary development plan advances previously approved plans, within the range and types of development already contemplated by the City of Lee’s Summit and Townsend Capital.  The introduction of the R-7 School District Summit Technology Academy (STA) and University of Central Missouri (UCM) strengthens the vision and innovation behind this public private partnership.  The anchor of this revised concept is the Missouri Innovation Campus – a 250,000 square foot education and research facility that will be located on 15 acres in the southeast portion of this plan area, along Tudor Road to be shared by STA and UCM.  This facility will be built in two phases to be identified more specifically in the subsequent final plan for that project.

Specifically, this plan update includes the updated rights-of-way for Tudor Road final design and construction and the eventual Ward Road expansion, new construction of innovation drive between Tudor and Ward Roads, 5 development parcels and anticipates phased build out of education, office, retail and hospitality uses that complement the range of uses in the surrounding areas.

UPDATED SUMMARY OF APPROVED DEVELOPMENT PLAN – SUMMIT INNOVATION CENTER

Project Description

- Context.  This plan update is part of a larger approved plan that includes The entire plan includes approximately 333 acres of land bordered on the north by Interstate 470, to the east by the Missouri Pacific Railroad, to the South by Chipman Road, and the west by NW Blue Parkway.  The portion immediately to the north of this plan update was recently approved for an updated PDP (Summit Place), and portions on the west side of the entire area were previously developed through an updated PDP (Summit Faire.)

- Area.   The currently approved plan is based on a series of green-space events that occur along the belt road, linking a series of villages that provide diverse settings for employment and commercial places themed under a “high-tech, sophisticated technology of contemporary office and business facilities.”  The landscape and street network that ties together the area is an extension of surrounding areas as well as building off of the original layout of the plan area.  “Midwestern landscape” and meandering tree plantings are envisioned to be added to the existing context to drive the character of the plan, and to tie the built plan into the surrounding context.

- Site /Projects.  The site and proposed projects exist at the northeast corner of the future Tudor Road construction (currently out for bid) and Ward Road (including its future expansion).  It borders on the Missouri Pacific Railroad to the east, and the previously approved Summit Place development plan to the north.

- Project information. This development plan envisions a mix of education / institution, office, retail (including general retail, restaurants, services, movie theater and fitness center) and hospitality uses across the almost 38 acre site.  The site will be anchored by the future Missouri Innovation Campus providing educational opportunities to high-school and college students. Access to the development from Wad Road will be supplemented with access from a new connection of Tudor Road from the east and the new Innovation Parkway to connect Ward Road to Tudor Road.

Site History

- April 13, 1999 – Lee’s Summit Planning Commission held a public hearing with due public notice in the manner prescribed by law regarding the rezoning of Summit Technology Campus from M-1 to PUD, and rendered a report to the City Council recommending approval of the rezoning.

- May 6, 1999 – Lee’s Summit City Council held a public hearing with due public notice in the manner prescribed by law and rendered a decision to rezoning Summit Technology Campus from M-1 to PUD. (The entire plan includes approximately 333 acres of land bordered on the north by Interstate 370, to the east by the Missouri Pacific Railroad, to the South by Chipman Road, and the west by NW Blue Parkway.)

- July 10, 2014 – Lee’s Summit City Council approved an updated Preliminary Development plan for the property to the north to facilitate development of Summit Place, consistent with the original approved overall preliminary development plan for Summit Technology Campus.

SITE DEVELOPMENT DATA

This development data provides specifications and requirements for development concepts illustrated in the site development plan.  These requirements shall control and guide future development in accordance with the site development plan.  Unless otherwise indicated in the site development plan, this data, or elsewhere in the narrative, all other provisions of the Lee’s Summit UDO will be used to guide development.

Table #1: Preliminary Development Plan Comparison

| Preliminary Development Plan Comparison |  |  |
| --- | --- | --- |
|  | Preliminary Development Concept (Illustrative) | Previously Approved Plan |
| Office | 90,000 | 322,000(1) |
| Retail | 103,000 | 56,000 (2) |
| Flex-Tech | -- | 234,000 (3) |
| Hospitality | 28,500 |  |
| Education / Institution | 250,000 + (90,000)(5) |  |
| Total Development | 471,500 | 612,000 sq. ft. |
| Parking | 2,298 Spaces (5.1 / 1,000 sq. ft.) | 2,276 Spaces(4) (3.72 /1,00 sq. ft.) |

(1) 	Previously approved plan includes the following buildings in this vicinity (O9, O12, O13, O14, O15, O16)

(2) 	Previously approved plan includes the following buildings in this vicinity (A1, and A2)

(3) 	Previously approved Plan includes the following buildings in this vicinity (F1, F2, F3)

(4) 	Previously approved plan was parked at an overall rate of 3.72 spaces per 1,000 sq.ft., based on a combination of generally 5.5/1K office, 5/1K retail, and approximately 1.6/1K for Flex-Tech.

(5)	The 90,000 sq. ft. identified in the illustrative plan as office use, could be developed as education/institution use with associated parking.

Site Development Standards

The site development concept prepared represents a development intensity less than that anticipated and approved by the previous preliminary development plan, see Table #1 for comparison.  The site development concept represents an illustrative example of the potential implementation of the Lee’s Summit Unified Development Ordinance development standards and the desired development uses, patterns, parking, landscaping and support elements for the Summit Innovation Center.  The site development standards included in this section are intended to provide flexibility and consistency in the future development of the overall development site and each lot, individually, within the development.

- Lots: The Summit Innovation Center will consist of 5 lots defined by the adoption of the preliminary plat information included and implemented by the approval of a final plat for each of the lots identified herein.  Each lot will be developed according to the standards provided herein, and with regard to adjacent lot development, if applicable.

- Land Uses: To expand the future development potential of the Summit Innovation Center a selection of appropriate uses has been identified for this development as well as each individual lot. The previously approved Preliminary Development Plan included office, retail and light industrial uses / flex-tech.  The uses identified as appropriate for future development includes office, shopping center (retail, restaurants, services, movie theaters, fitness centers and other commercial establishments), hospitality, education and institution uses.  Education and institution uses represent new uses to the mix of uses in this area, as office, retail and hospitality uses were approved within the previous development plan.  The inclusion of education and institution uses is important for the future development of the Missouri Innovation Campus and supporting uses.  Uses specific to individual lots within the Development Site are identified in Table #2.

- Development Plan Standards (Table #2): The plan standards for land area, pervious coverage, floor area, floor-area-ratio, height and land uses are defined in Table #2. The development standards have been prepared for each individual lot within the Summit Innovation Center.

Table #2: Development Plan Standards

| Development Plan Standards |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Lot | Land Area (ac.) | Minimum Pervious Coverage | Maximum Floor Area | Maximum FAR | Maximum Height | Allowable Land Uses |
| 2 | 9.77 | 5% | 97,750 | .4 | Up to 4 stories or 60 feet in height | Shopping Center Office Education / Institution |
| 3 | 2.14 | 5% | 47,500 | .4 | Up to 4 stories or 60 feet in height | Hospitality Retail |
| 4 | 5.56 | 5% | 125,000 | .5 | Up to 4 stories or 60 feet in height | Office Education Institution Shopping Center |
| 5 | 5.19 | 5% | 50,000 | .4 | Up to 4 stories or 60 feet in height. | Shopping Center |
| 6 | 15.0 | 5% | 281,250 | .5 | Up to 4 stories or 60 feet in height. | Education / Institution Office |
| Total PDP | 37.66 | 5% | 601,500 | .44 (avg.) | 4 stories / 60 feet |  |

- Parking (Table #3): To ensure that parking is adequately provided for the development site and each lot within, the parking requirements defined in Table 3 will be applied to all future development within the Summit Innovation Center.  The parking calculations are based on the requirements of the Unified Development Ordinance, unique uses and anticipated use patterns of the development.  To meet the UDO requirements for number of parking space for any one building, parking may need to be shared between lots within the Summit Innovation Campus.  If sharing does occur the minimum requirements of the UDO for each specific use will meet the requirements as outlined in Table #3: Parking Standards. ADA accessible parking spaces will meet the minimum parking code requirements for all development, individual lots and the development site.

Educational / Institutional Use Parking – The spaces necessary for the education / institutional uses are anticipated to be college teaching classrooms, laboratory space and an auditorium.  It is anticipated that the parking necessary to support these uses will be 3 spaces per 1,000 square feet of space, based on the following justification:

Article 12 of the City of Lee’s Summit’s Unified Development Ordinance identifies the following minimum parking requirements:

- College Instructional Space - 10 stalls for each classroom

- Technical College and Trade Schools – 20 stalls for each classroom

- Senior High Schools – 6 stalls for each classroom

Places of Assembly – 1 stall for every 3 seats.

The initial phase of the educational / institutional space is anticipated to be approximately 150,000 square feet, to include 27 college classrooms, 13 high school class rooms and a 200 set auditorium.  Use the UDO requirements for those uses we would need to provide 415 parking spaces and at 3 spaces per thousand we would need to provide 450 parking spaces, and excess of 35 parking spaces.  The space defined on the preliminary development plan for the Lot 6 educational / institutional uses is 250,000 square feet, requiring 750 parking spaces and we have identified a capacity of more than 1,000 parking spaces on that lot.  In other locations where educational / institutional uses may be developed, the 3 spaces per 1,000 square feet will be met.

Table #3: Parking Standards

| Parking Standards |  |  |
| --- | --- | --- |
| Land Use | Parking Standards (/ 1,000 sq. ft. of building) | UDO Parking Requirements (Article 12) |
| Educational / Institution | 3* | Mix of uses defined. |
| Office | 4 | 4 |
| Shopping Center (retail, restaurant, | 5 | 5 |
| Hospitality | 1 (/ guest room) | 1 (/ guest room) |

*Provides parking for a mixture of college teaching classrooms, laboratory space and an auditorium.

The previously approved Preliminary Development Plan modified the parking stall size to 9’ wide x 18’ deep from the Unified Development Ordinance required size of 9’ wide by 19’ deep.  This preliminary development plan preserves that modification to allow a minimum size of parking stall at 9’ wide by 18’ deep.  The site development concept included in this submission illustrates the code required parking stall size.

- Pervious Surface / Landscape Design: Future development of the Summit Innovation Center will meet, and in many cases exceed the pervious surface requirement (as defined in Table #2) and landscape concept, as defined by Sheet L100 Landscape Concept, and those of the Lee’s Summit Unified Development Code.  The landscape concept provided is an illustrative example of the landscape that is anticipated with development of the site. While it is expected that the species of plant materials will adhere to those identified in the landscape concept, the location and spacing of materials on individual sites may differ to provide landscape and plantings that support and enhance the development of that site.

- Site Access and Circulation: Primary access to development within the Summit Innovation Center will occur from Ward Road, the new Tudor Road and the new Innovation Drive, all public streets.  Access points will meet the Lee’s Summit Unified Development Ordinance requirements for driveways and access.  Similarly, internal circulation, including driveways, drive aisles, radii and connectivity, within the Summit Innovation Center, will meet code requirements unless further defined within this preliminary development plan.

- Building Design:

The Missouri Innovation Campus will be a regional destination and the character of that facility will provide the guidance for the future character of the site development.  The Missouri Innovation Campus is currently beginning the design process.  As such, the character of that facility and in turn the character of the site has not yet been defined.  However, we are committed to a quality development with a defining, consistent character.

Sheet A200 Exterior Aesthetics represents the palette of building materials that are anticipated for development of the Summit Innovation Center.  As the Missouri Innovation Campus is further designed, this palette will be refined for the continued development of the site.  The intent is to provide a consistent character of design across the entire development site that reflects that uniqueness and style defined by the Missouri Innovation Campus building.  A spirit sketch of the proposed Missouri Innovation Campus building has been included to illustrate the unique design potential of that building and its ability to set the design character for the rest of the site.  While materials across the development site may vary, yet be consistent with the palette of materials shown, the character of the development will strive for consistency.

Mechanical Screening

Mechanical screening of equipment will meet requirements of the UDO.

- Signs and Lighting Design: The signage and lighting locations provided on the site development plan are illustrative in nature and the final locations will be determined by the development of the site and individual lots.  Lighting will be designed and located in such a manner to meet the UDO standards.

Signage - The signage on site is anticipated to follow the Unified Development Ordinance standards including:

- 1 monument or landscape wall sign per building (72 s.f. sign face; 96 s.f. structure);

- Building signs:  3 per building or 2 per tenant; 10% of façade; 6 foot max letter height

However, the distance between monument signs (75’ per the UDO) may not be met dependent on the final location and siting of individual buildings. A modification to this requirement for this sign location is anticipated.

This preliminary development plan anticipates the need for additional signs larger than those allowed by Chapter 13 of the Unified Development Ordinance, because of the unique nature of the Missouri Innovation Campus and potential support uses.  Therefore, an allowance for two additional signs that may not conform to ordinance requirements for size, lighting, placement and other requirements are incorporated as part of this preliminary development plan.  These signs will further the unique character of design and the uses that will be developed as part of the Summit Innovation Center.

Lighting – General lighting locations have been identified on the site development concept. However, the final location and design of those fixtures will be determined as development occurs.  The design of the lighting across the site will be in character with the design of the buildings on site and will be consistent across the site.  Similar to the design character for the site, the character of lighting and other site elements will be informed by the design of the Missouri Innovation Campus building and site.

WAIVERS / REGULATIONS

Waivers from a few of the Unified Development Ordinance standards were adopted with the previous preliminary development plan adopted, per Section 2.  1.3.a.- d. of Ordinance No. 4793.  Those waivers are included with this new preliminary development plan and will be used in the design of the development site and individual lots where appropriate.

Previously approved waivers.

- A waiver for sidewalks on only one side of the Ward road where the hike and bike trail runs parallel to the road to link the park and pond areas.

Other Regulations

- Changes to this preliminary development plan, that are not in excess of 25 % of the defined standards, are acceptable and will be approved by administrative measures when submitted to the City for approval.   In particular the building size or footprint size may change up to 25% as long as the other design items as defined in this preliminary development plan are met.

PHASING

The anticipated phasing of development for the Summit Innovation Center is described in Table #4.

Table 4: Phasing Plan

| Phasing Plan |  |
| --- | --- |
| Phase | Development |
| Phase 1 | Lot 6 |
| Phase 2 | Lot 3 |
| Phase 3 | Lots 2, 4 & 5 (in no particular order) |